- VoiceTronix P/L

- 6/13 Bode Ave, North Wollongong NSW 2500 AUSTRALIA

- Phone: +61 2 4227 2174 Fax: +61 2 4227 2371

- email: voicet@ozemail.com.au

# VPB4 Software Installation Procedure for NT 4.0

- Unzip the downloaded VPB4_nt_install_v1.1.zip into a floppy disk using winzip. This will be called the VPB4 NT4.0 series installation disk.

- Place the VPB4 NT4.0 series installation disk in your floppy drive.

- Go to the Start Toolbar and click on Run.

- Type a:\install to run the install program with its default settings. The default settings assume the floppy drive is located at a:\. This copies the installation files to c:\vpb. If you would like to install the software from or to another drive (different from the default settings), then Type sourcedrive:\install destdrive:\subdirectory\ sourcedrive:\ (eg. a:\install k:\vpb\ a:\ ) This copies the installation files from drive a:\ to the subdirectory vpb in drive k:\

- After successfully completing the installation of the software, open VPB.hlp in the \vpb directory on your PC for complete notes on installing the VPB4 card. This may be done by double clicking on the VPB.hlp file under the \vpb directory using explorer.

- Under the contents tab of the VPB.hlp there is an "installation" topic. Double click on it and then click on the "hardware installation" heading. Here you will find a sub-heading labeled "To install the Hardware". Follow the instructions.  The software that has been installed assumes that the PC has the I/O base address (0x300) free. If there is another device on the PC using the default base address then you can change the DIP switch settings that set the base address. Once this is done go to the "Software Installation" topic in VPB.hlp and double click "Setting the Base Address under Windows NT" topic.

- Follow the instructions and set the base address in the windows registry to the appropriate value defined by the DIP switch settings on the hardware.

- Once this is complete reboot the PC (if you have not already done so since adding the VPB4 hardware and software driver).

- In the \vpb directory of your root drive there is a program executable called tcomm.exe. Running this will test the VPB4 hardware. It conducts memory checks, downloads the firmware and communication tests between the PC and the VPB4 card. One should see a PING and PONG count being updated. Press any key to exit the program.

- If an error occurs and the program doesn't run, it usually signifies a base address conflict. A message appears stating that COFF_DSP_MEMORY_TEST_FAILED. To change the base address follow step 5 & 6. There is no need to reboot the PC, just change the base address settings on the hardware to a new setting, as depicted in the VPB.hlp file under "To install the Hardware" and in the registry as illustrated in "Setting the Base Address under Windows NT" topic.

- It may be useful to use the Windows NT Diagnostics program, click on the Resources tab and then the I/O Port to view used base addresses in the system. This program may be found when you click the Start toolbar, go to programs, then administrative tools. This assists in finding a spare base address. Note 0x300 to 0x3f0 can be used as base address, in 0x10 steps (eg. 0x300, 0x310…0x3f0).

# Examples

Included in the installation disk are various examples (source code) that demonstrate the use of the API functions. These include examples for the VPB8L and VPB4 card. Check with the VPB.hlp file for the VPB4 supported API functions. Look under the "function reference" topic for the appropriate API function.

## Installation of Examples

1. There is a zip file in the \examples directory of the VPB4 NT4.0 series installation disk. Copy to a directory in your PC (eg. c:\vpb\examples). Unzip the file using winzip to find examples of API functions.

2. The examples are console applications and have been compiled and executed successfully using Microsoft Visual C++ 4.2.

3. Each file is a self-contained program that illustrates the use of an API function.

Note: Make sure the vpbapi.lib (default location c:\vpb) module is linked to the application.

# Contact Details

For customer support or inquiries, the contact details are:

Tel. +61 2 4227 2174

Fax. +61 2 4227 2371

Email. voicet@ozemail.com.au

Web Site. www.users.bigpond.com/voicet